# Needlepoint Canvas Painter

Software Development Specification

Version 1.0 - December 2024

# 1. System Overview

The application generates G-code files to paint needlepoint canvas patterns using the WorkBii pen plotter. It converts pattern images into optimized toolpaths with automatic pen changes.

## 1.1 Core Functions

- Import needlepoint pattern images (PNG, JPG, PDF)

- Map pattern colors to available pen colors

- Generate canvas hole coordinates based on mesh count

- Optimize paint order to minimize pen changes and travel

- Output G-code compatible with Klipper firmware

- Support pause/resume for pen replacement

## 1.2 Hardware Interface

| Component | Interface |
| --- | --- |
| ODROID-N2+ | Klipper host, runs Moonraker API |
| SKR Pico | Klipper MCU, executes G-code |
| Stepper Motors | X, Y, Z axes via TMC2209 drivers |
| Pen Lift Servo | MG90D on PWM pin, controlled via SET_SERVO |
| Gripper Servo | MG996R on PWM pin, tool change macro |
| Limit Switches | Pen detection in rack slots |

# 2. Input Formats

## 2.1 Pattern Image Requirements

| Property | Requirement |
| --- | --- |
| Format | PNG (preferred), JPG, PDF, BMP |
| Color Mode | RGB or indexed color |
| Resolution | 1 pixel = 1 canvas hole (ideal) |
| Max Colors | Limited by pen rack (8-12 colors) |
| Background | Transparent or designated "no paint" color |

## 2.2 Canvas Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| mesh_count | 18 | Holes per inch (14, 18, 22 common) |
| canvas_width_mm | 300 | Physical canvas width |
| canvas_height_mm | 300 | Physical canvas height |
| origin_x_mm | 50 | X offset from machine origin |
| origin_y_mm | 50 | Y offset from machine origin |
| hole_diameter_mm | 0.8 | Target paint dot size |

## 2.3 Mesh to MM Conversion

Canvas mesh count defines hole spacing:

hole_spacing_mm = 25.4 / mesh_count

Example: 18-mesh = 25.4 / 18 = 1.411mm between hole centers

| Mesh | Hole Spacing | Holes in 300mm |
| --- | --- | --- |
| 14-mesh | 1.814mm | 165 x 165 |
| 18-mesh | 1.411mm | 212 x 212 |
| 22-mesh | 1.155mm | 260 x 260 |

# 3. Coordinate System

## 3.1 Machine Coordinates

- Origin (0,0) at front-left corner after homing

- X axis: left to right (0 to ~550mm)

- Y axis: front to back (0 to ~770mm)

- Z axis: up/down, 0 = pen touching canvas

- All units in millimeters

## 3.2 Canvas Coordinates

Pattern pixels map to canvas holes:

machine_x = origin_x + (pixel_col * hole_spacing)

machine_y = origin_y + (pixel_row * hole_spacing)

## 3.3 Pen Rack Coordinates

Pen rack positioned at rear of work area:

| Slot | X Position | Notes |
| --- | --- | --- |
| T0 | 10mm | First pen slot |
| T1 | 40mm | 30mm spacing |
| T2 | 70mm |  |
| T3-T11 | 100-340mm | Up to 12 positions |
| Y Position | 750mm | Rear of work area |
| Z (pickup) | 5mm | Lower gripper to this Z |

# 4. G-code Reference

## 4.1 Essential Commands

| Command | Description |
| --- | --- |
| G28 | Home all axes |
| G90 | Absolute positioning mode |
| G91 | Relative positioning mode |
| G0 X_ Y_ F_ | Rapid move (pen up) |
| G1 X_ Y_ F_ | Linear move (pen down) |
| G1 Z_ F_ | Z axis move |
| G4 P_ | Dwell (pause) in milliseconds |
| M400 | Wait for moves to complete |
| M112 | Emergency stop |

## 4.2 Klipper-Specific Commands

| Command | Description |
| --- | --- |
| SET_SERVO SERVO=pen ANGLE=90 | Lift pen (up position) |
| SET_SERVO SERVO=pen ANGLE=45 | Lower pen (down position) |
| SET_SERVO SERVO=gripper ANGLE=90 | Open gripper |
| SET_SERVO SERVO=gripper ANGLE=45 | Close gripper |
| T0, T1, T2... | Tool change (calls macro) |
| PAUSE | Pause print (pen empty) |
| RESUME | Resume after pause |
| SAVE_GCODE_STATE NAME=x | Save current position |
| RESTORE_GCODE_STATE NAME=x | Restore saved position |

## 4.3 Feed Rates

| Operation | Speed | F Value |
| --- | --- | --- |
| Rapid travel (pen up) | 100mm/s | F6000 |
| Painting move | 50mm/s | F3000 |
| Z movement | 20mm/s | F1200 |
| Tool change approach | 30mm/s | F1800 |

# 5. Paint Sequence

## 5.1 Single Hole Paint Cycle

Each canvas hole requires this sequence:

- Move to hole position (G0 X_ Y_ with pen up)

- Lower pen (SET_SERVO SERVO=pen ANGLE=45)

- Dwell for paint transfer (G4 P50-100)

- Raise pen (SET_SERVO SERVO=pen ANGLE=90)

- Move to next hole

## 5.2 Example G-code Snippet

; Paint hole at (100, 150)

G0 X100 Y150 F6000      ; Rapid to position

SET_SERVO SERVO=pen ANGLE=45  ; Pen down

G4 P75                  ; Dwell 75ms

SET_SERVO SERVO=pen ANGLE=90  ; Pen up

; Next hole...

## 5.3 Tool Change Sequence

When switching pen colors:

; Switch from T0 to T2

T2                      ; Calls tool change macro

; Macro internally does:

;   SAVE_GCODE_STATE NAME=tool_change

;   G90

;   SET_SERVO SERVO=pen ANGLE=90

;   G0 X10 Y750 F6000   ; Move to T0 slot

;   SET_SERVO SERVO=gripper ANGLE=90  ; Open

;   G1 Z5 F1200         ; Lower

;   G4 P300

;   G1 Z30 F1200        ; Raise (pen stays)

;   G0 X70 Y750 F6000   ; Move to T2 slot

;   G1 Z5 F1200         ; Lower

;   SET_SERVO SERVO=gripper ANGLE=45  ; Close

;   G4 P300

;   G1 Z30 F1200        ; Raise with pen

;   RESTORE_GCODE_STATE NAME=tool_change

# 6. Path Optimization

## 6.1 Goals

- Minimize total travel distance

- Minimize tool changes (group by color)

- Avoid dragging wet paint

- Maintain consistent pen pressure

## 6.2 Recommended Strategy

- Group all holes by color

- Sort colors by frequency (most common first)

- For each color, optimize path using nearest-neighbor or TSP

- Insert tool change commands between color groups

## 6.3 Path Algorithms

| Algorithm | Notes |
| --- | --- |
| Nearest Neighbor | Simple, fast. Visit closest unvisited hole next. ~25% longer than optimal. |
| 2-opt | Improves existing path by swapping segments. Good refinement step. |
| Scanline | Paint row by row, alternating direction. Predictable, easy to verify. |
| Hilbert Curve | Space-filling curve. Good locality, moderate complexity. |

Recommendation: Start with scanline for simplicity. Add nearest-neighbor + 2-opt later if needed.

# 7. Data Structures

## 7.1 Core Types

interface Point {

x: number;      // Machine X in mm

y: number;      // Machine Y in mm

}

interface Hole {

row: number;    // Pattern row (0-indexed)

col: number;    // Pattern column

color: Color;   // Assigned color

position: Point; // Machine coordinates

painted: boolean;

}

interface Color {

id: string;     // Unique identifier

name: string;   // Display name

rgb: [number, number, number];

toolIndex: number; // T0, T1, T2...

}

interface Job {

name: string;

canvas: CanvasConfig;

colors: Color[];

holes: Hole[];

toolpath: Point[];  // Optimized order

}

# 8. Application Architecture

## 8.1 Recommended Stack

| Layer | Technology |
| --- | --- |
| Frontend | React + TypeScript, Canvas for preview |
| Backend | Node.js or Python (FastAPI) |
| Image Processing | Sharp (Node) or Pillow (Python) |
| G-code Generation | Custom TypeScript/Python module |
| Machine Communication | Moonraker API (REST + WebSocket) |

## 8.2 Core Modules

- PatternParser: Load image, extract color map, generate hole list

- ColorMapper: Match pattern colors to available pens

- PathOptimizer: Order holes to minimize travel

- GcodeGenerator: Convert toolpath to G-code

- MoonrakerClient: Send jobs, monitor progress

- JobManager: Save/load jobs, track progress

## 8.3 Moonraker API Endpoints

| Endpoint | Purpose |
| --- | --- |
| POST /printer/gcode/script | Send G-code commands |
| POST /server/files/upload | Upload G-code file |
| POST /printer/print/start | Start print job |
| POST /printer/print/pause | Pause print |
| POST /printer/print/resume | Resume print |
| GET /printer/objects/query | Get printer status |
| WS /websocket | Real-time status updates |

# 9. Phase 2: Vision Integration

## 9.1 Vision Capabilities

- Canvas hole detection (align pattern to physical canvas)

- Color verification (confirm correct pen is loaded)

- Paint coverage check (detect skipped holes)

- Pen exhaustion detection (monitor paint output)

## 9.2 OpenCV Algorithms

| Task | Method |
| --- | --- |
| Hole Detection | SimpleBlobDetector with size/circularity filters |
| Color Check | HSV comparison in ROI around pen tip |
| Coverage Analysis | Threshold + contour area vs expected |
| Canvas Alignment | findHomography with detected corners |

## 9.3 Vision Event Handlers

// Pseudo-code for vision callbacks

onPenExhausted(toolIndex) {

pausePrint();

notifyUser('Replace pen in slot ' + toolIndex);

}

onColorMismatch(expected, detected) {

pausePrint();

notifyUser('Wrong pen color detected');

}

onSkippedHole(hole) {

addToRetryQueue(hole);

}

# 10. Output File Format

## 10.1 G-code File Structure

; Header

; Generated by Needlepoint Canvas Painter

; Pattern: [filename]

; Canvas: [mesh]mesh, [width]x[height]mm

; Colors: [count]

; Total holes: [count]

; Estimated time: [minutes]

; Initialization

G28                 ; Home all

G90                 ; Absolute mode

SET_SERVO SERVO=pen ANGLE=90

T0                  ; Load first pen

; Color 1: Red (T0) - 1234 holes

; ... paint commands ...

; Color 2: Blue (T1) - 987 holes

T1                  ; Tool change

; ... paint commands ...

; Finish

SET_SERVO SERVO=pen ANGLE=90

G0 X0 Y0 F6000      ; Return home

M400                ; Wait for completion

# 11. Klipper Configuration

## 11.1 printer.cfg Excerpts

[servo pen]

pin: gpio29

initial_angle: 90

[servo gripper]

pin: gpio28

initial_angle: 45

[gcode_macro T0]

gcode:

SAVE_GCODE_STATE NAME=tool_change

G90

SET_SERVO SERVO=pen ANGLE=90

{% if printer.toolhead.tool != 0 %}

; Return current pen

G0 X{printer.toolhead.tool * 30 + 10} Y750 F6000

SET_SERVO SERVO=gripper ANGLE=90

G1 Z5 F1200

G4 P300

G1 Z30 F1200

{% endif %}

; Pick up T0

G0 X10 Y750 F6000

G1 Z5 F1200

SET_SERVO SERVO=gripper ANGLE=45

G4 P300

G1 Z30 F1200

SET_GCODE_VARIABLE MACRO=TOOL_STATE VARIABLE=tool VALUE=0

RESTORE_GCODE_STATE NAME=tool_change

# 12. Testing Checklist

- Verify coordinate system matches machine (home, move to corners)

- Test pen lift/lower timing (adjust dwell if paint smears)

- Calibrate tool change positions (fine-tune X/Y/Z for each slot)

- Test gripper grip force (adjust servo angle if pen slips)

- Run small test pattern before full canvas

- Verify hole spacing matches physical canvas mesh

- Monitor servo temperature during long runs

- Test pause/resume mid-job

# 13. Reference Links

Klipper Documentation: klipper3d.org

Moonraker API: moonraker.readthedocs.io

G-code Reference: reprap.org/wiki/G-code

OpenCV Python: docs.opencv.org