# WorkBii Vision Plotter

Complete Parts List - Two Phase Build

Updated: December 29, 2024

# Project Overview

Purpose: Vision-enabled pen plotter for needlepoint canvas painting with automatic pen changer

Frame: WorkBii 750x1000mm CNC kit

Working Area: ~550mm x 770mm (21.6" x 30.3")

Resolution: 0.1mm (400 steps/mm)

Architecture: Klipper (ODROID-N2+ host + SKR Pico MCU)

# Phase 1: Plotter + Pen Changer

Complete functional plotter with 8-12 position automatic pen changer. Timeline: 6-8 weeks.

## Order 1: Hardkernel (ORDERED)

| Component | Qty | Price | Link |
| --- | --- | --- | --- |
| ODROID-N2+ 4GB | 1 | $88.00 | hardkernel.com |
| ODROID-N2 Case Black | 1 | $4.00 | hardkernel.com |
| 32GB eMMC Module N2 Linux | 1 | $19.00 | hardkernel.com |
| 12V/2A Power Supply US Plug | 1 | $5.50 | hardkernel.com |
| Hardkernel Subtotal: |  |  | $116.50 |

Status: Ordered. Ships from Korea, 1-2 weeks.

## Order 2: BulkMan3D

| Component | Qty | Price | Link |
| --- | --- | --- | --- |
| WorkBii 750x1000mm CNC Kit + Motors | 1 | $400.00 | bulkman3d.com |
| BulkMan3D Subtotal: |  |  | $400.00 |

Note: Ships from China, 2-4 weeks. Order early - longest lead time.

## Order 3: Amazon

| Component | Qty | Price | Link |
| --- | --- | --- | --- |
| Motion Control |  |  |  |
| BTT SKR Pico | 1 | $25.00 | Amazon |
| TMC2209 Stepper Drivers | 4 | $20.00 | Amazon |
| 24V 15A Power Supply (Mean Well) | 1 | $25.00 | Amazon |
| Pen System |  |  |  |
| MG90D Digital Servo (pen lift) | 1 | $12.00 | Amazon |
| MG996R Servo (gripper) | 1 | $12.00 | Amazon |
| Servo Extension Cables 300mm | 4 | $8.00 | Amazon |
| 3D Printed Pen Mount (PETG) | 1 | $5.00 | Self-print |
| Compression Springs 10mm OD | 2 | $3.00 | Amazon |
| Pen Changer (8-12 positions) |  |  |  |
| 3D Printed Pen Rack (PETG) | 1 | $10.00 | Self-print |
| 3D Printed Gripper (PETG) | 1 | $5.00 | Self-print |
| Limit Switches (pen detect) | 4 | $8.00 | Amazon |
| 3mm Steel Rods (gripper rails) | 2 | $5.00 | Amazon |
| Wiring & Electrical |  |  |  |
| 22AWG Stranded Wire (Red/Black) | 1 | $12.00 | Amazon |
| JST-XH Connector Kit | 1 | $15.00 | Amazon |
| Ferrule Crimps (22-18 AWG) | 1 | $10.00 | Amazon |
| IEC C14 Panel Mount Inlet | 1 | $8.00 | Amazon |
| Rocker Switch (illuminated) | 1 | $5.00 | Amazon |
| Hardware |  |  |  |
| M3 x 8mm Socket Head Screws | 20 | $5.00 | Amazon |
| M3 T-Nuts (for V-slot) | 20 | $5.00 | Amazon |
| M5 T-Nuts (for V-slot) | 10 | $5.00 | Amazon |
| Debug & Tools |  |  |  |
| DSD TECH SH-U09C USB-UART Adapter | 1 | $10.00 | Amazon |
| Amazon Subtotal: |  |  | $208.00 |

Note: Order after frame ships. 2-5 day delivery.

## Order 4: Local / Misc

| Component | Qty | Price | Source |
| --- | --- | --- | --- |
| MDF Spoilboard 750x1000x12mm | 1 | $25.00 | Home Depot |
| PETG Filament (pen mount, rack) | 0.5kg | $15.00 | Amazon/Local |
| Local Subtotal: |  |  | $40.00 |

## Phase 1 Cost Summary

| Category | Cost |
| --- | --- |
| Hardkernel (ODROID-N2+) | $116.50 |
| BulkMan3D (WorkBii 750x1000mm) | $400.00 |
| Amazon (electronics, hardware) | $208.00 |
| Local (spoilboard, filament) | $40.00 |
| PHASE 1 TOTAL: | $764.50 |

# Phase 2: Vision System

Camera system for hole detection, color verification, and autonomous operation. Timeline: 4-6 weeks after Phase 1.

| Component | Qty | Price | Link |
| --- | --- | --- | --- |
| IMX477 12.3MP Camera Module | 1 | $60.00 | Amazon |
| CS-Mount Lens (8-12mm) | 1 | $25.00 | Amazon |
| Camera Ribbon Cable (extended) | 1 | $10.00 | Amazon |
| LED Ring Light (diffused) | 1 | $15.00 | Amazon |
| 3D Printed Camera Mount | 1 | $5.00 | Self-print |
| USB-C to USB-A cable (camera) | 1 | $8.00 | Amazon |
| Phase 2 Subtotal: |  |  | $123.00 |

# Project Total

| Phase | Cost |
| --- | --- |
| Phase 1: Plotter + Pen Changer | $764.50 |
| Phase 2: Vision System | $123.00 |
| GRAND TOTAL: | $887.50 |

# Order Checklist

- Hardkernel - ODROID-N2+ kit (ORDERED)

- BulkMan3D - WorkBii 750x1000mm frame (order now, longest lead time)

- Amazon - SKR Pico, TMC2209, servos, wiring, USB-UART

- Local - MDF spoilboard, PETG filament

- Amazon - Vision components (after Phase 1 complete)

# Key Component Notes

## WorkBii 750x1000mm Frame

- Larger than original 750x750mm plan

- Working area ~550mm x 770mm

- More space for pen rack placement

- No rigidity issues for pen plotting (no cutting forces)

## MG90D Digital Servo (Pen Lift)

- Digital servo for high-cycle pen lifting

- 20% faster than analog MG90S

- ~100-150K cycle life

- Better position holding under load

## DSD TECH SH-U09C USB-UART Adapter

- Genuine FTDI FT232RL chip

- 3.3V/5V selectable (use 3.3V for ODROID)

- Serial console for debugging boot issues

- TX/RX LEDs for troubleshooting

## Klipper Architecture

- ODROID-N2+ runs Klipper host + OpenCV (Phase 2)

- SKR Pico handles real-time stepper control

- Solves Linux real-time limitation

- Web interface via Mainsail/Fluidd

# Important Links

WorkBii Kit: bulkman3d.com

Hardkernel: hardkernel.com

USB-UART Adapter: Amazon - DSD TECH SH-U09C

Klipper Docs: klipper3d.org